General formula and process

The two input parameters

N_ADC        = measured ADC noise floor    (linear, from FFT median)

T_dB         = target system noise above ADC noise    (e.g. 4 dB)

Step 1 — Convert target to linear

T_lin = 10^(T_dB / 10)

For T_dB = 4:

T_lin = 10^(4/10) = 2.511

Step 2 — Compute target RF noise at ADC input

System noise is the sum of ADC noise and RF chain noise:

N_system = N_ADC + N_RF

You want N_system = T_lin × N_ADC, so:

N_RF_target = (T_lin - 1) × N_ADC

For the example:

N_RF_target = (2.511 - 1) × 1 = 1.511

Step 3 — Measure current RF noise at your starting gain

At your current AGC setting G_current, measure N_system via FFT median, then:

N_RF_current = N_system_current - N_ADC

Step 4 — Compute required gain reduction

Noise power scales linearly with gain power, so:

ΔG_dB = 10 × log10( N_RF_current / N_RF_target )

Step 5 — Set new AGC gain

G_target = G_current - ΔG_dB

Full formula in one block

T_lin        = 10^(T_dB / 10)

N_RF_target  = (T_lin - 1) × N_ADC

N_RF_current = N_system_current - N_ADC

ΔG_dB        = 10 × log10( N_RF_current / N_RF_target )

G_target     = G_current - ΔG_dB

The software process — how to automate it

Given that your software can set AGC gain, record FFT, and compute median, the procedure is:

Phase 1 — Measure ADC noise baseline

Set AGC to minimum gain (or RF input terminated if possible). Record FFT, compute median across bins. This is N_ADC[ch] for each channel. Store it — this is your calibration baseline and should only be remeasured if hardware changes.

Phase 2 — Measure system noise at a known high gain

Set AGC to a known high gain G_current (e.g. maximum 35 dB). Record FFT, compute median. This is N_system_current[ch] per channel.

Phase 3 — Compute and apply target gain per channel

For each channel independently:

T_lin           = 10 ** (T_dB / 10)

N_RF_target     = (T_lin - 1) * N_ADC[ch]

N_RF_current    = N_system_current[ch] - N_ADC[ch]

delta_G         = 10 * log10(N_RF_current / N_RF_target)

G_target[ch]    = G_current - delta_G

Apply G_target[ch] to each channel's AGC.

Phase 4 — Verify

After applying the new gain, measure FFT median again per channel and confirm:

N_system_verify[ch] ≈ N_ADC[ch] * T_lin    # within ±0.5 dB tolerance

If a channel is out of tolerance, run one correction iteration:

N_RF_residual   = N_system_verify[ch] - N_ADC[ch]

delta_G_trim    = 10 * log10(N_RF_residual / N_RF_target)

G_target[ch]   -= delta_G_trim

One iteration is almost always sufficient. The process converges because the relationship between gain and noise power is linear.

Important boundary checks in your software

# RF chain must be adding noise — if not, gain is already too low

if N_RF_current <= 0:

raise Error("N_system <= N_ADC on channel ch — check measurement")

# RF noise must already exceed target — if not, even max gain is insufficient

if N_RF_current < N_RF_target:

raise Warning("Channel ch: RF noise below target even at current gain

— system is ADC-noise limited at this gain setting")

# Resulting gain must be within AGC hardware limits

if G_target[ch] < G_min or G_target[ch] > G_max:

raise Error("Computed gain out of AGC range on channel ch")

Numerical example with your numbers and T_dB = 4

N_ADC          = 1

T_dB           = 4

G_current      = 35 dB

N_system       = 4

T_lin          = 2.511

N_RF_target    = 1.511

N_RF_current   = 4 - 1 = 3

ΔG_dB          = 10 × log10(3 / 1.511) = 10 × log10(1.986) = 2.98 dB

G_target       = 35 - 2.98 = 32.02 dB

Round to your AGC step resolution and verify. Done.